Electrostatic problems, solved with BELA - Electrostatics

Contributed by Kostadin Brandisky

# Problem 1:    Capacitance of line with cylindrical conductors

Wire diameter =75 mm,

Distance between the wires d=200 mm.

The line is in air with =0 .

The capacitance per unit length is sought.

d



The model is in the file line2.fee   . The mesh has  9799 nodes. Kelvin transformation is used for the infinite boundary.

The capacitance will be calculated using the stored electric energy.

C=2Wεu2

Stored energy, found by FEMM is :

Wε=8,50624.10−12 J

The capacitance will be:

C=2Wεu2=17,0125.10−12 Fm

Here   u=(V2 - V1)= 1 – 0 = 1

The analytical formula for C is :

C=π ε llnd2R+d2R2−1=16,99.10−12 Fm

където:   = 0 = 8,8541878.10-12 F/m,  l=1 m,  d=0,2 m,  R=0,0375 m.

|  | Analytical | FEMM | Error % |
| --- | --- | --- | --- |
| C | 16,99.10-12 | 17,0125.10-12 | 0,13 |

# Problem 2:    Microstrip line (problem elec1 from Quickfield)

Air   r=1.

Substrate with r=10 .

The capacitance per unit length is sought.

On the metal strip a boundary condition V=1 V is specified. On the outher conductor V=0.

The model is in   elec1.fee   having   40178  nodes.

The stored energy, found by FEMM is:

Wε=89,8428.10−12

Thus

C=2Wεu2=179,6856.10−12 Fm

Here   u=(V2 - V1)= 1 – 0 = 1

|  | Analytical result | QuickField pF/m | Error % | FEMM pF/m | Error % |
| --- | --- | --- | --- | --- | --- |
| C | 178,1 | 177,33 | 0,4 | 179,6856 | 0,9 |